# Orion Controller - Power Subsystem Requirements

This document captures the binding requirements for the Orion industrial controller power subsystem. Any regulator selected for the 3.3 V logic rail must satisfy every requirement in Table 1.

## Table 1: Binding Requirements

| Requirement ID prefix | ORN-PWR |
| --- | --- |
| Logic rail voltage | 3.3 V plus or minus 2 percent |
| Minimum continuous current | 2.0 A |
| Maximum output ripple | 20 mV pp |
| Required qualification | AEC-Q100 Grade 1 |
| Required supply commitment | 10 years minimum |
| Maximum lead time | 16 weeks |
| Ambient temperature range | -40 C to +125 C |

## Notes

The 20 mV ripple limit is driven by the 16-bit ADC on the sensor board. Requirement ORN-PWR-014 states that any switching regulator must be followed by post-regulation if this limit cannot be met directly.